《嵌入式系统新技术及应用》实验报告

| 姓名 | 徐志鹏 | 实验日期 | 2026.4.13 | 实验成绩 |  |
| --- | --- | --- | --- | --- | --- |
| 班级 | 电A2361 | 学号 | 12 |  |  |
| 实验名称 | 实验一 按键控制LED和蜂鸣器实验 |  |  | 教师签字 |  |

一、实验目的

1、掌握MDK软件的安装。

2、掌握GPIO的编程应用。

二、实验原理

STM32单片机的GPIO有8种工作模式，其中数字输入有GPIO_Mode_IN_FLOADING、GPIO_Mode_IPU、GPIO_Mode_IPD三种，数字输出有GPIO_Mode_OUT_OD、GPIO_Mode_OUT_PP、GPIO_Mode_AF_OD、GPIO_Mode_AF_PP四种，利用GPIO可检测数字输入信号，还可以作数字控制，或模拟I2C、SPI等通信接口。

三、实验内容

根据下面的电路用STM32F103C8T6实现功能：

1.按键1~4分别控制LED1~4的亮暗。按键按下时LED灯亮，松下即暗。

2.按下按键时，蜂鸣器给出0.5秒的提示音。

四、实验结果记录

1.引脚功能分配

| F103C8T6引脚 | 控制板引脚 | 功能 | 初始值 | 备注 |
| --- | --- | --- | --- | --- |
| PA0 | KEY1 | Input | N/A |  |
| PA2 | KEY2 | Input | N/A |  |
| PA4 | KEY3 | Input | N/A |  |
| PA6 | KEY4 | Input | N/A |  |
| PB12 | BUZ | Out Push Pull | H |  |
| PB5 | LED1 | Out Push Pull | H |  |
| PB6 | LED2 | Out Push Pull | H |  |
| PB7 | LED3 | Out Push Pull | H |  |
| PB8 | LED4 | Out Push Pull | H |  |

2.控制方法描述：

按键控制（PA0/PA2/PA4/PA6）

配置：设为GPIO输入，启用内部上拉(Out Push Pull)

读取：轮询或中断读取电平，低电平为按下

消抖：检测到变化后延时10-20ms再确认

LED控制（PB5/PB6/PB7/PB8）

重映射：开启GPIOB时钟

配置：设为推挽输出，初始输出高电平（熄灭）

控制：低电平点亮，高电平熄灭

蜂鸣器控制（PB12）

配置：设为推挽输出，初始输出高电平（关闭）

控制：低电平鸣叫

3.实验结果描述（文字和图片）：

按键按下时对应的LED灯亮，松下即暗；按下时，蜂鸣器给出0.5秒的提示音。

五、心得体会

通过本次STM32按键控制实验，我主要有以下几点收获：

- 掌握GPIO配置：深刻理解了输入与输出模式的区别。按键需配置为上拉输入以避免电平悬空，而LED和蜂鸣器则需配置为推挽输出以提供足够的驱动能力。

- 理解软件消抖：认识到机械按键存在抖动现象，通过软件延时（如20ms）进行二次确认，是保证按键检测准确性的关键。

- 硬件驱动逻辑：明白了单片机IO口驱动能力有限，同时区分了有源蜂鸣器（给电平即响）与无源蜂鸣器的使用差异。

- 编程思维提升：通过模块化编程（将按键、LED、蜂鸣器分开封装），让主程序逻辑更加清晰，体会到了良好代码结构对开发效率的提升。

六、自己编写的程序

1. main()函数

/* USER CODE BEGIN Header */

/**

******************************************************************************

* @file           : main.c

* @brief          : Main program body

******************************************************************************

* @attention

*

* Copyright (c) 2026 STMicroelectronics.

* All rights reserved.

*

* This software is licensed under terms that can be found in the LICENSE file

* in the root directory of this software component.

* If no LICENSE file comes with this software, it is provided AS-IS.

*

******************************************************************************

*/

/* USER CODE END Header */

/* Includes ------------------------------------------------------------------*/

#include "main.h"

#include "gpio.h"

/* Private includes ----------------------------------------------------------*/

/* USER CODE BEGIN Includes */

#include "stdbool.h"

/* USER CODE END Includes */

/* Private typedef -----------------------------------------------------------*/

/* USER CODE BEGIN PTD */

typedef struct {

GPIO_TypeDef* KEY_PORT;

uint16_t KEY_PIN;

GPIO_TypeDef* LED_PORT;

uint16_t LED_PIN;

} KeyLedMap_t;

/* USER CODE END PTD */

/* Private define ------------------------------------------------------------*/

/* USER CODE BEGIN PD */

// 按键消抖延时（ms）

#define KEY_DELAY_MS    20

// 蜂鸣器鸣响时长（ms）

#define BUZZER_BEEP_MS  100

/* USER CODE END PD */

/* Private macro -------------------------------------------------------------*/

/* USER CODE BEGIN PM */

#define READ_KEY(PORT, PIN)  (HAL_GPIO_ReadPin(PORT, PIN) == GPIO_PIN_RESET)

/* USER CODE END PM */

/* Private variables ---------------------------------------------------------*/

/* USER CODE BEGIN PV */

// 按键-LED映射表

const KeyLedMap_t KeyLedMap[] = {

{GPIOA, KEY_1_Pin, GPIOB, LED_1_Pin},

{GPIOA, KEY_2_Pin, GPIOB, LED_2_Pin},

{GPIOA, KEY_3_Pin, GPIOB, LED_3_Pin},

{GPIOA, KEY_4_Pin, GPIOB, LED_4_Pin}

};

// 按键数量

#define KEY_LED_NUM    (sizeof(KeyLedMap)/sizeof(KeyLedMap_t))

// 蜂鸣器状态标记

bool buzzer_beep_flag = false;

uint32_t buzzer_beep_start_time = 0;

/* USER CODE END PV */

/* Private function prototypes -----------------------------------------------*/

void SystemClock_Config(void);

/* USER CODE BEGIN PFP */

void Key_Scan_Handler(void);

void Buzzer_Control(void);

/* USER CODE END PFP */

/* Private user code ---------------------------------------------------------*/

/* USER CODE BEGIN 0 */

/**

* @brief  按键扫描处理函数（消抖+LED控制）

* @param  None

* @retval None

*/

void Key_Scan_Handler(void)

{

static uint8_t key_press_flag = 0;

// 遍历所有按键

for(uint8_t i=0; i<KEY_LED_NUM; i++)

{

// 按键消抖：连续读取两次状态一致则判定为有效

if(READ_KEY(KeyLedMap[i].KEY_PORT, KeyLedMap[i].KEY_PIN))

{

HAL_Delay(KEY_DELAY_MS);

if(READ_KEY(KeyLedMap[i].KEY_PORT, KeyLedMap[i].KEY_PIN))

{

// 按键按下：点亮对应LED，触发蜂鸣器

HAL_GPIO_WritePin(KeyLedMap[i].LED_PORT, KeyLedMap[i].LED_PIN, GPIO_PIN_RESET);

key_press_flag = 1;

}

}

else

{

// 按键松开：熄灭对应LED

HAL_GPIO_WritePin(KeyLedMap[i].LED_PORT, KeyLedMap[i].LED_PIN, GPIO_PIN_SET);

}

}

// 有按键按下时触发蜂鸣器

if(key_press_flag)

{

buzzer_beep_flag = true;

buzzer_beep_start_time = HAL_GetTick();

key_press_flag = 0;

}

}

/**

* @brief  蜂鸣器控制函数

* @param  None

* @retval None

*/

void Buzzer_Control(void)

{

if(buzzer_beep_flag)

{

// 打开蜂鸣器

HAL_GPIO_WritePin(GPIOB, Buzzing_Pin, GPIO_PIN_RESET);

// 达到鸣响时长后关闭蜂鸣器

if(HAL_GetTick() - buzzer_beep_start_time >= BUZZER_BEEP_MS)

{

HAL_GPIO_WritePin(GPIOB, Buzzing_Pin, GPIO_PIN_SET);

buzzer_beep_flag = false;

}

}

else

{

// 默认关闭蜂鸣器

HAL_GPIO_WritePin(GPIOB, Buzzing_Pin, GPIO_PIN_SET);

}

}

/* USER CODE END 0 */

/**

* @brief  The application entry point.

* @retval int

*/

int main(void)

{

/* USER CODE BEGIN 1 */

/* USER CODE END 1 */

/* MCU Configuration--------------------------------------------------------*/

/* Reset of all peripherals, Initializes the Flash interface and the Systick. */

HAL_Init();

/* USER CODE BEGIN Init */

/* USER CODE END Init */

/* Configure the system clock */

SystemClock_Config();

/* USER CODE BEGIN SysInit */

/* USER CODE END SysInit */

/* Initialize all configured peripherals */

MX_GPIO_Init();

/* USER CODE BEGIN 2 */

HAL_GPIO_WritePin(GPIOB, LED_1_Pin|LED_2_Pin|LED_3_Pin|LED_4_Pin, GPIO_PIN_SET);

HAL_GPIO_WritePin(GPIOB, Buzzing_Pin, GPIO_PIN_SET);

/* USER CODE END 2 */

/* Infinite loop */

/* USER CODE BEGIN WHILE */

while (1)

{

/* USER CODE END WHILE */

/* USER CODE BEGIN 3 */

// 按键扫描

Key_Scan_Handler();

// 蜂鸣器控制

Buzzer_Control();

}

/* USER CODE END 3 */

}

/**

* @brief System Clock Configuration

* @retval None

*/

void SystemClock_Config(void)

{

RCC_OscInitTypeDef RCC_OscInitStruct = {0};

RCC_ClkInitTypeDef RCC_ClkInitStruct = {0};

/** Initializes the RCC Oscillators according to the specified parameters

* in the RCC_OscInitTypeDef structure.

*/

RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSE;

RCC_OscInitStruct.HSEState = RCC_HSE_ON;

RCC_OscInitStruct.HSEPredivValue = RCC_HSE_PREDIV_DIV1;

RCC_OscInitStruct.HSIState = RCC_HSI_ON;

RCC_OscInitStruct.PLL.PLLState = RCC_PLL_ON;

RCC_OscInitStruct.PLL.PLLSource = RCC_PLLSOURCE_HSE;

RCC_OscInitStruct.PLL.PLLMUL = RCC_PLL_MUL9;

if (HAL_RCC_OscConfig(&RCC_OscInitStruct) != HAL_OK)

{

Error_Handler();

}

/** Initializes the CPU, AHB and APB buses clocks

*/

RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_HCLK|RCC_CLOCKTYPE_SYSCLK

|RCC_CLOCKTYPE_PCLK1|RCC_CLOCKTYPE_PCLK2;

RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_PLLCLK;

RCC_ClkInitStruct.AHBCLKDivider = RCC_SYSCLK_DIV1;

RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV2;

RCC_ClkInitStruct.APB2CLKDivider = RCC_HCLK_DIV1;

if (HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_2) != HAL_OK)

{

Error_Handler();

}

}

/* USER CODE BEGIN 4 */

/* USER CODE END 4 */

/**

* @brief  This function is executed in case of error occurrence.

* @retval None

*/

void Error_Handler(void)

{

/* USER CODE BEGIN Error_Handler_Debug */

/* User can add his own implementation to report the HAL error return state */

__disable_irq();

while (1)

{

}

/* USER CODE END Error_Handler_Debug */

}

#ifdef  USE_FULL_ASSERT

/**

* @brief  Reports the name of the source file and the source line number

*         where the assert_param error has occurred.

* @param  file: pointer to the source file name

* @param  line: assert_param error line source number

* @retval None

*/

void assert_failed(uint8_t *file, uint32_t line)

{

/* USER CODE BEGIN 6 */

/* User can add his own implementation to report the file name and line number,

ex: printf("Wrong parameters value: file %s on line %d\r\n", file, line) */

/* USER CODE END 6 */

}

#endif /* USE_FULL_ASSERT */

2.Gpio.c

/* USER CODE BEGIN Header */

/**

******************************************************************************

* @file    gpio.c

* @brief   This file provides code for the configuration

*          of all used GPIO pins.

******************************************************************************

* @attention

*

* Copyright (c) 2026 STMicroelectronics.

* All rights reserved.

*

* This software is licensed under terms that can be found in the LICENSE file

* in the root directory of this software component.

* If no LICENSE file comes with this software, it is provided AS-IS.

*

******************************************************************************

*/

/* USER CODE END Header */

/* Includes ------------------------------------------------------------------*/

#include "gpio.h"

/* USER CODE BEGIN 0 */

/* USER CODE END 0 */

/*----------------------------------------------------------------------------*/

/* Configure GPIO                                                             */

/*----------------------------------------------------------------------------*/

/* USER CODE BEGIN 1 */

/* USER CODE END 1 */

/** Configure pins as

* Analog

* Input

* Output

* EVENT_OUT

* EXTI

*/

void MX_GPIO_Init(void)

{

GPIO_InitTypeDef GPIO_InitStruct = {0};

/* GPIO Ports Clock Enable */

__HAL_RCC_GPIOD_CLK_ENABLE();

__HAL_RCC_GPIOA_CLK_ENABLE();

__HAL_RCC_GPIOB_CLK_ENABLE();

/*Configure GPIO pin Output Level */

HAL_GPIO_WritePin(GPIOB, Buzzing_Pin|LED_1_Pin|LED_2_Pin|LED_3_Pin

|LED_4_Pin, GPIO_PIN_SET);

/*Configure GPIO pins : PAPin PAPin PAPin PAPin */

GPIO_InitStruct.Pin = KEY_1_Pin|KEY_2_Pin|KEY_3_Pin|KEY_4_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_INPUT;

GPIO_InitStruct.Pull = GPIO_PULLUP;

HAL_GPIO_Init(GPIOA, &GPIO_InitStruct);

/*Configure GPIO pins : PBPin PBPin PBPin PBPin

PBPin */

GPIO_InitStruct.Pin = Buzzing_Pin|LED_1_Pin|LED_2_Pin|LED_3_Pin

|LED_4_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_HIGH;

HAL_GPIO_Init(GPIOB, &GPIO_InitStruct);

}

/* USER CODE BEGIN 2 */

/* USER CODE END 2 */